Compliance Program

29 CFR 1926.60—Methylenedianiline

29 CFR 1926.62—Lead

29 CFR 1926 Subpart Z—Toxic and Hazardous Substances

Scope/Application: Under 29 CFR 1926.60—Methylenedianiline, 29 CFR 1926.62—Lead, and most of the health standards listed in 29 CFR Part 1926 Subpart Z—Toxic and Hazardous Substances, a Compliance Program along with other programs is required if employees have occupational exposure to that chemical. In addition to Methylenedianiline and Lead, this includes the following standards within Subpart Z:

- 29 CFR 1926.60—Methylenedianiline

- 29 CFR 1926.62—Lead

- 29 CFR 1926.1117—Vinyl Chloride

- 29 CFR 1926.1118—Inorganic Arsenic

- 29 CFR 1926.1124—Beryllium

- 29 CFR 1926.1127—Cadmium

- 29 CFR 1926.1128—Benzene

- 29 CFR 1926.1144—1,2—dibromo—3—chloropropane

- 29 CFR 1926.1145—Acrylonitrile

- 29 CFR 1926.1147—Ethylene Oxide

- 29 CFR 1926.1152—Methylene Chloride

- 29 CFR 1926.1153—Respirable Crystalline Silica

Note: Many of the health standards found in 29 CFR 1926 Subpart Z—Toxic and Hazardous Substances refer back to the applicable health standard in 29 CFR 1910 Subpart Z—Toxic and Hazardous Substances as being identical for construction work.

The following standards are referenced in 29 CFR 1926.60—Methylenedianiline, 29 CFR 1926.62—Lead, and most of the health standards listed in Subpart Z—Toxic and Hazardous Substances:

- 29 CFR 1926.33—Access to Employee Exposure and Medical Records

- 29 CFR 1926.51—Sanitation

- 29 CFR 1926.59—Hazard Communication

- 29 CFR 1926.103—Respiratory Protection

- 29 CFR 1910.134—Respiratory Protection

- 29 CFR 1910.141—Sanitation

- 29 CFR 1910.1020—Access to Employee Exposure and Medical Records

- 29 CFR 1910.1200—Hazard Communication

Note: 29 CFR 1926.33—Access to Employee Exposure and Medical Records (Refers to 29 CFR 1910.1020—Access to Employee Exposure and Medical Records) requires that medical and exposure records be maintained for specific periods of time. 29 CFR 1926 Subpart Z—Toxic and Hazardous Substances also requires that showers and change rooms meet the criteria found in 29 CFR 1926.51—Sanitation or 29 CFR 1910.141—Sanitation. 29 CFR 1926.59—Hazard Communication 29 CFR 1910.1200—Hazard Communication requirements apply when the applicable health hazard is used in the workplace. 29 CFR 1926.103 (Refers to 1910.134—Respiratory Protection) requirements apply when employees use respirators.

Standard Requirements for 29 CFR 1926.60—Methylenedianiline:

- Programs/Plans: Compliance Program, Medical Surveillance Program, Training Program, Housekeeping and Inspection Program, Employer-Funded Compensation Program

- Procedures/Practices: Exposure monitoring procedures (Include in your Compliance Program)

- Training: Initially, annually

- Inspections: Routine, regular

- Documentation: Programs, medical records, exposure records

Note: Reference 29 CFR 1926.60—Methylenedianiline to ensure that all requirements are begin met.

Standard Requirements for 29 CFR 1926.62—Lead:

- Programs/Plans: Compliance Program, Medical Surveillance Program, Training Program, Work Practice Program, Employer-Funded Compensation Program

- Procedures/Practices: Work practice controls, exposure monitoring procedures, medical surveillance procedures (Include in your Compliance Program)

- Training: Initially, annually

- Inspections: Frequent, regularly (Reference 1926.20—General Safety and Health Provisions)

- Documentation: Programs, medical records, exposure records

Note: Reference 29 CFR 1926.62—Lead to ensure that all requirements are being met.

Standard Requirements for 29 CFR 1926.1117—Vinyl Chloride (Same as 29 CFR 1910.1017):

- Programs/Plans: Exposure Monitoring Program, Emergency Operational Plan, Medical Surveillance Program, Training and Indoctrination Program

- Procedures/Practices: Work practice controls, exposure monitoring procedures, medical surveillance procedures (Include in your Compliance Program)

- Training: Initially, annually

- Inspections: Frequent, regularly (Reference 1926.20—General Safety and Health Provisions)

- Documentation: Programs, medical records, exposure records, training records

Note: The standard does not require a Compliance Program but as a best practice, the employer should consider implementing one. Please reference 29 CFR 1926.1117—Vinyl Chloride to ensure all requirements are being met.

Standard Requirements for 29 CFR 1926.1118—Inorganic Arsenic (Same as 29 CFR 1910.1018):

- Programs/Plans: Compliance Program, Medical Surveillance Program, Housekeeping and Maintenance Plan, Training Program

- Procedures/Practices: Work practice controls, exposure monitoring procedures, medical surveillance procedures (Include in your Compliance Program)

- Training: Initially, annually

- Inspections: Frequent, regularly (Reference 1926.20—General Safety and Health Provisions)

- Documentation: Programs, medical records, exposure records

Note: Reference 29 CFR 1926.1118—Inorganic Arsenic to ensure that all requirements are being met.

Standard Requirements for 29 CFR 1926.1124—Beryllium:

- Programs/Plans: Exposure Control Plan (Compliance Program), Medical Surveillance Program, Employer-Funded Compensation Program

- Procedures/Practices: Work practice controls, engineering controls, safety and health procedures, medical surveillance procedures (Include in your Compliance Program)

- Training: Initially, annually

- Inspections: Frequent, regularly

- Documentation: Plan, medical records, training records, exposure records

Note: Reference 29 CFR 1926.1124—Beryllium to ensure that all requirements are being met.

Standard Requirements for 29 CFR 1926.1127—Cadmium:

- Programs/Plans: Compliance Program, Medical Surveillance Program, Emergency Plan, Training Program

- Procedures/Practices: Work practice controls, exposure monitoring procedures, medical surveillance procedures (Include in your Compliance Program)

- Training: Initially, annually

- Inspections: Frequent, regularly (Reference 1926.20—General Safety and Health Provisions)

- Documentation: Programs, medical records, exposure records

Note: Reference 29 CFR 1926.1127—Cadmium to ensure that all requirements are being met.

Standard Requirements for 29 CFR 1926.1128—Benzene (Same as 29 CFR 1910.1028):

- Programs/Plans: Compliance Program, Medical Surveillance Program, Training Program, Employer-Funded Compensation Program

- Procedures/Practices: Work practice controls, exposure monitoring procedures, medical surveillance procedures (Include in your Compliance Program)

- Training: Initially, annually

- Inspections: Frequent, regularly (Reference 1926.20—General Safety and Health Provisions)

- Documentation: Programs, medical records, exposure records

Note: Reference 29 CFR 1926.1128—Benzene to ensure that all requirements are being met.

Standard Requirements for 29 CFR 1926.1144—1,2-dibromo-3-chloropropane (Same as 29 CFR 1910.1044):

- Programs/Plans:  Compliance Program, Medical Surveillance Program, Training Program

- Procedures/Practices: Work practice controls, exposure monitoring procedures, medical surveillance procedures (Include in your Compliance Program

- Training: Initially, annually

- Inspections: Frequent, regularly (Reference 1926.20—General Safety and Health Provisions)

- Documentation: Programs, medical records, exposure records

Note: Reference 29 CFR 1926.1144—1,2-dibromo-3-chloropropane to ensure that all requirements are being met.

Standard Requirements for 29 CFR 1926.1145—Acrylonitrile (Same as 29 CFR 1910.1045):

- Programs/Plans:  Compliance Program, Medical Surveillance Program, Training Program, Emergency Plan

- Procedures/Practices: Work practice controls, exposure monitoring procedures, medical surveillance procedures (Include in your Compliance Program)

- Training: Initially, annually

- Inspections: Frequent, regularly (Reference 1926.20—General Safety and Health Provisions)

- Documentation: Programs, medical records, exposure records

Note: Reference 29 CFR 1926.1145—Acrylonitrile to ensure that all requirements are being met.

Standard Requirements for 29 CFR 1926.1147—Ethylene Oxide (Same as 29 CFR 1910.1047):

- Programs/Plans: Compliance Program, Medical Surveillance Program, Training Program

- Procedures/Practices: Work practice controls, exposure monitoring procedures, medical surveillance procedures (Include in your Compliance Program)

- Training: Initially, annually

- Inspections: Frequent, regularly (Reference 1926.20—General Safety and Health Provisions)

- Documentation: Programs, medical records, exposure records

Note: Reference 29 CFR 1926.1147—Ethylene Oxide to ensure that all requirements are being met.

Standard Requirements for 29 CFR 1926.1152—Methylene Chloride (Same as 29 CFR 1910.1052:

- Programs/Plans: Employee Exposure Program, Employer-Funded Compensation Program, Medical Surveillance Program

- Procedures/Practices: Work practice controls, exposure monitoring procedures, medical surveillance procedures, leak detection procedures

- Training: Initially, annually

- Inspections: Frequent, regularly (Reference 1926.20—General Safety and Health Provisions)

- Documentation: Medical records, exposure records, training information

Note: Reference 29 CFR 1926.1152—Methylene Chloride to ensure that all requirements are being met.

Standard Requirements for 29 CFR 1926.1153—Respirable Crystalline Silica:

- Programs/Plans: Exposure Control Plan (Compliance Program), Medical Surveillance Program

- Procedures/Practices: Work practice controls, engineering controls, exposure monitoring procedures, medical surveillance procedures, work entry procedures

- Training: Initially

- Inspections: Frequent, regularly

- Documentation: Medical records, exposure records, plan

Note: Reference 29 CFR 1926.1153—Respirable Crystalline Silica to ensure that all requirements are being met.

Example Program: The following example program may be modified to be site-specific to the organization and incorporates a Medical Surveillance Program, Exposure Monitoring Program and Training Program with the Compliance Program. Please reference the applicable health standard to ensure that all requirements are being met.

Toxic and Hazardous Substances—Compliance Program

______________ can cause harm (describe acute and chronic effects of the chemical) to:

__________________________________________________________________________________________________________________________________________________________________________________________

_____________________ is used (state where this chemical is used and what it is used for):

__________________________________________________________________________________________________________________________________________________________________________________________

Engineering Controls

The following engineering controls have been implemented:

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Work Practices

The following work practices have been implemented:

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Regulated Areas, Signs and Labels

We will regulate the work areas where _______________ is used. These regulated areas will be limited to authorized employees only and will be marked with signage. The signs will bear the following legend:

DANGER
__________________________

PROTECTIVE CLOTHING ARE REQUIRED IN
THIS AREA
AUTHORIZED PERSONNEL ONLY

All ____________________ containers and pipes have been labeled. The Safety Data Sheets will also be available in the work area (or other procedure).

Preventive Maintenance Program

Preventive maintenance will be conducted on the ventilation systems as recommended by the manufacturer.  This schedule is as follows:

____________________________________________________________________________________________________________________________________________________________________________________

Air filters will be replaced on the following schedule:

____________________________________________________________________________________________________________________________________________________________________________________

Other preventive maintenance recommended by the manufacturer will be conducted on the following equipment or machines:

____________________________________________________________________________________________________________________________________________________________________________________

This schedule is as follows:

____________________________________________________________________________________________________________________________________________________________________________________

Personal Protective Equipment

Employees will be provided with appropriate personal protective equipment (PPE) which will include:

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

All employees will be required to wear the appropriate PPE in the regulated area. The PPE will be cleaned and inspected daily and before each use. Any equipment found to be defective will be turned in to the supervisor for repair or disposal and new equipment will be provided. All employees will be trained on the use of PPE.

Employees required to wear respirators will undergo medical evaluation to determine if they can wear a respirator. Employees who pass will be trained and fit-tested initially and annually per 29 CFR 1926.103—Respiratory Protection (or 29 CFR 1910.134—Respiratory Protection). Refer to the respirator program for specific requirements.

Signs and Symptoms of Overexposure

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

If you exhibit any of the above signs, please let your supervisor know immediately.

First Aid and Emergency Response

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Spills

- Call 911 and HazMat

- Immediately evacuate all personnel

- Secure and control entrance to facility

- Eliminate nearby ignition sources

Emergency Numbers

- Poison Control: 1-800-222-1222

- CHEMTREC: 1-800-424-9300

- National Response Center: 1-800-424-8802

- Company Emergency Contact:

Note: Some of the standards in Subpart Z may have additional or different emergency response requirements.

Exposure Monitoring Program

________________________ is a regulated chemical. To help ensure that our employees are not overexposed to__________________, we will conduct ________________(i.e., initial, periodic) monitoring to determine employee exposure.

Note: Reference the applicable standard for the monitoring schedule and exposure levels to be followed.

The OSHA permissible exposure level (PEL) for _________________ is_________ and the action level is_______.

When exposure levels are at or above the (action level) of ________ (i.e., parts per million, milligrams per cubic meter), periodic monitoring will be conducted on the following schedule (i.e., quarterly, semi-annually, annually):

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Determination of employee exposure will be made from breathing zone air samples that are representative of the 8-hour TWA and 15-minute short-term exposures of each employee. Employees will be provided the monitoring results within 15 working days after receipt of results. If requested, employees will be allowed to observe exposure monitoring.

Medical Surveillance Program

If an employee is exposed to ______________ at or above the action level of  ______ (i.e., parts per million, milligrams per cubic meter), they will be provided with medical examinations and consultations at no expense to the employee. The examinations will be provided prior to an assignment where exposure will occur and annually thereafter. If required by a physician, the examinations may occur more frequently.

We will provide the following information to the physician:

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

The following tests will be conducted initially and annually:

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

The physician will provide a written opinion containing the results of the medical examination and any recommended limitations on the employee or on the use of PPE. No specific findings or diagnoses unrelated to _______________ exposure will be given in the written opinion.  The employee will be provided with a copy of the written opinion within 15 days of its receipt.

Housekeeping Program

We have implemented the following housekeeping procedures:

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Training Program

Employees will be trained at the time of their initial assignment and annually thereafter. Training will include:

- Operations where “chemical” is present

- Signs and symptoms of overexposure

- Safe work practices

- Established or recommended exposure levels

- Exposure monitoring

- Medical surveillance

- PPE

- Physical and health hazards

- Methods and observations

- Emergency procedures

- Hazard communication

Note: Some of the standards in 29 CFR 1926 Subpart Z—Toxic and Hazardous Substances and 29 CFR 1910 Subpart Z—Toxic and Hazardous Substances may have additional or different training requirements.

Recordkeeping

Medical and exposure records will be maintained per 29 CFR 1926.33—Access to Employee Exposure and Medical Records (or 1910.1020—Access to Employee Exposure and Medical Records). Exposure records will be maintained for 30 years. Medical surveillance records will be maintained for the duration of employment plus 30 years. Employees will be provided these records upon request.

Note: Some of the standards in 29 CFR 1926 Subpart Z—Toxic and Hazardous Substances and 29 CFR 1910 Subpart Z—Toxic and Hazardous Substances may have different durations for maintaining records.